File 1 — run_on_demand_20hz_timed.py

What this file is for (big-picture)

- This program runs an experiment that:

1. Records video from a camera (or more than one camera).

2. Sometimes triggers a small external device (PulsePal) to send a repeating light pulse at 20 times per second.

3. Shows a simple window with buttons so a person can start the pulse, mark when an “attack” stopped, and stop/save the video and logs.

- It also writes files to disk:

- A video file (the recorded images).

- A CSV file with times when pulses started and ended and whether they were on.

- A text file with camera timestamps.

Main pieces

- PulsePalObject (PulsePal): this talks to the PulsePal.

- PySpin: the camera company’s library the code uses to talk to the camera.

- cv2: the part that handles showing an image window and writing video files.

- GUI window (tkinter): a small window that has three buttons the person uses:

- “Run Pulse Train” — starts the 20 Hz light pulses.

- “Record Attack Stop” — writes down when the person says an “attack” stopped.

- “Stop and Save” — stops recording and saves all files.

User questions the program asks at the start

- folder_name: Where, on the computer, you want the saved files put.

- name_file: The base name for the video and log files (you choose it).

- format_file: The video file format (for example, avi or mp4).

- camera_time: How long you want to record (seconds). If you leave it blank, it uses 600 seconds (10 minutes).

Settings

- SERIAL_PORT = 'COM4'

- TOTAL_DURATION_SECONDS = 10

- How long the light-pulse sequence should run when you start it. In the code it’s 10 seconds.

- FRAME_RATE_HZ = 20

- Camera frame rate. Here it should be 20.

- BLUE = 2

- Uses channel 2 on the PulsePal for the blue light.

- PULSE_VOLTAGE_BLUE = 5.0

- How strong the voltage sent to the light should be (a measure of brightness/power).

- PULSE_FREQUENCY_HZ_BLUE = 20

- How fast the pulses repeat per second (20 Hz).

- ON_DURATION_SECONDS_BLUE and OFF_DURATION_SECONDS_BLUE

- How long each pulse is ON and how long it is OFF. In this case ON and OFF are each 1/40 second.

- time_rn, year_month_day

- Time-stamped text added to filenames so saved files are unique and include the date/time.

- log_folder_name = 'logs' and camera_log_folder_name = 'camera_logs'

- Subfolders to keep the log files organized.

Video-saving settings

- frame_width = 720 and frame_height = 540

- The picture size in pixels (width and height).

- fourcc = cv2.VideoWriter_fourcc(*'XVID')

- Method used to compress and save video.

- video_writer = cv2.VideoWriter(...)

- The object that writes the frames (pictures) into a file on disk as a video.

Files created and where

- video_file_path: full path for the video file (folder/name + timestamp + extension).

- log_file_path: full path for the CSV file with pulse/timing information.

- camera_log_file_path: full path for a text file with camera-to-computer timestamp info.

Lists that store things during the session

- time_log, camera_timelog, start_times, attack_stop_times, end_times, on_status

- Puts time and status information while the experiment is running, then later writes them to files.

When you press the “Run Pulse Train” button:

- The program randomly decides whether to actually turn on the pulse device or not (equally as likely)

- It starts a separate “background worker” that connects to the PulsePal device, sets the channel settings (voltage, on/off durations, total length), and tells the PulsePal to start sending pulses.

- While PulsePal runs, that thread sleeps for the duration and then records the start and end times (and whether the device was actually turned on) into the lists for logging.

Camera recording and threads:

- For each camera the user allows, the program:

- Initializes the camera.

- Sets it up so that each image carries its own timestamp inside it.

- Starts a separate worker (thread) that reads images from the camera, displays them in a window, and sends them to the video file. Each thread returns a list of camera timestamps when it is done.

Buttons and stopping behavior

- “Run Pulse Train” — start the pulse

- “Record Attack Stop” — when a mouse attack ends, press this button so the program saves that time in attack_stop_times. It should only be pressed once per pulse train.

- “Stop and Save” — tells the camera threads to stop, stops polling for pulse results, waits until everything is finished, and then saves the video and log files.

After everything finishes, the program:

- Closes the video file (so the movie is properly saved).

- Turns recorded times into a table and writes them as a CSV file.

- Writes camera timestamps into the camera log file(s).

- Shuts down the cameras and ends the program.

----------------------------

File 2 — CameraTimeToPCTime.py

What this file is for:

- Cameras store times for each picture using their internal camera clock. The computer (PC) also has its own clock. These two clocks can differ.

- This file contains code that:

- Figures out how to convert a camera’s internal time into the computer’s time.

- Records images from the camera, converts each image’s camera timestamp into a PC timestamp, and returns those converted timestamps.

- Allows us to match pulse/attack times with the exact frames in the video.

Key functions:

- is_newer(cam_name)

- Checks whether this camera is in the “newer” category (meaning it uses one conversion method) or not.

- calculate_offset_older_usb(cam), calculate_offset_older_gev(cam), calculate_offset_newer(cam)

- Each of these tries to compute the difference (offset) between the camera’s clock and the PC clock.

- setup_chunk_data(cam)

- Tells the camera to include metadata with each image that contains the camera timestamp.

- acquire_images(cam, writer, height, width, num_frames, frame_rate_hz)

- This is the main capture loop:

- It sets the camera to a specific frame rate (for example 20 frames per second).

- It figures out which offset calculation method to use (newer or older).

- It starts the camera taking pictures.

- For each picture:

- It gets the picture data and shows it on the screen.

- It writes the picture into the video file.

- It reads the timestamp the camera attached to that picture.

- It converts that timestamp into PC time using the offset and records it as a readable date/time.

- When done it returns the list of converted PC timestamps for the captured frames.

- run_single_camera(cam) and main()

- Helpers to run the process for every connected camera.

----------------------------

Putting the two files together:

- run_on_demand_20hz_timed.py uses functions from CameraTimeToPCTime.py to:

- Set up the camera so each image has a timestamp chunk.

- Start capturing images and get the list of converted timestamps for those images.

- Meanwhile, run_on_demand_20hz_timed.py can also command the PulsePal device to emit a 20 Hz pulse train and logs when that pulse started and ended.

- With everything logged in PC times, you can compare the exact times the lights were on with the exact frames in the video.